TECHNICAL SERVICE REPORT

Water Analysis

| Customer: | Location: | Date: | PO#: |
| --- | --- | --- | --- |
| Chick-fil-A | # 1110 | 12/28/06 | 265412 |
| Address: |  | Tested By: |  |
| Jefferson Village FSU 9976 W. Remington Place Littleton, CO 80128 |  | AR |  |
| Phone: | Email: | Selecto Contact: |  |
| 720-922-2070 |  |  |  |

| Recommendation: |  |
| --- | --- |

##### ANALYTICAL RESULTS

| Evaluation of: | Men’s Restroom Water Sample | Ladies Restroom Water Sample | Recommended Max Levels |
| --- | --- | --- | --- |
| Fluoride, ppm | 0.8 | 0.8 | 2 ppm |
| Chloride, ppm | 50 | 52 | 250 ppm |
| Nitrite, ppm | ND | ND | 1 ppm as N |
| Nitrate, ppm | 0.3 | 0.3 | 10 ppm as N |
| Sulfate, ppm | 49 | 49 | 250 ppm |
| Sodium, ppm | 30 | 31 | 250 ppm |
| Ammonia as Chloramine | Detected | Detected | .5 ppm |
| Chlorine, ppm | Detected | Detected | Detectable |
| Alkalinity, ppm | 50 | 55 | 150 ppm |
| Total Dissolved Solids, ppm | 190 | 190 | 500 ppm |
| Hardness, ppm | 74 | 71 | 100 ppm |
| pH | 7.3 | 7.6 | 6.5 – 8.5 ppm |
| Turbidity | 0.00 | 0.00 | .05 NTU |

The information supplied is believed to be accurate and is only indicative of the water tested.  The testing laboratory is not state certified.

3980 Lakefield Court    Suwanee, GA 30024    678.475.0799    800.635.4017

www.selectoinc.com sales@selectoinc.com