TECHNICAL SERVICE REPORT

Water Analysis

| Customer: | Location: | Date: | PO#: |
| --- | --- | --- | --- |
| Bruce Dale | # 1883 | 10/25/05 | 228944 |
| Address: |  | Tested By: |  |
| Harrisonburg FSU 1691 E. Market Street Harrisonburg, VA 22801 |  | AR |  |
| Phone: | Email: | Selecto Contact: |  |
| 770-883-1933 |  | John Edmonds |  |

| Recommendation: | Scheduled to Open January 2006 |
| --- | --- |

##### ANALYTICAL RESULTS

| Evaluation of: | Wendy’s Water Sample | Waffle House Water Sample | Recommended Max Levels |
| --- | --- | --- | --- |
| Fluoride, ppm | 1.2 | 1.2 | 2 ppm |
| Chloride, ppm | 22 | 20 | 250 ppm |
| Nitrite, ppm | ND | ND | 1 ppm as N |
| Nitrate, ppm | 1.2 | 1.2 | 10 ppm as N |
| Sulfate, ppm | 13 | 13 | 250 ppm |
| Sodium, ppm | 26 | 24 | 10 ppm |
| Ammonia as Chloramine | Must be tested on site | Must be tested on site | .5 ppm |
| Chlorine, ppm | Must be tested on site | Must be tested on site | Detectable |
| Alkalinity, ppm | 85 | 90 | 150 ppm |
| Total Dissolved Solids, ppm | 150 | 150 | 500 ppm |
| Hardness, ppm | 92 | 87 | 100 ppm |
| pH | 7.5 | 7.5 | 6.5 – 8.5 ppm |
| Turbidity | 1.4 | 0.75 | 0.5 NTU |
| Particle Counts, #/cc | 1,084 (4-63 micron) | 1,078 (8-63 micron) |  |

The information supplied is believed to be accurate and is only indicative of the water tested.  The testing laboratory is not state certified.

3980 Lakefield Court    Suwanee, GA 30024    678.475.0799    800.635.4017

www.selectoinc.com

sales@selectoinc.com